Table 4.1: Actions included in Scenario 1

| Subsector | Transformation name | Description | Magnitude |
| --- | --- | --- | --- |
| Agriculture | Improve rice management | Improve rice cultivation practices to reduce methane emissions by 45%, including better water and fertilizer management, appropriate variety selection, residue management, and optimal seeding. This transformation directly reduces the rice-specific CH4 emission factor. | 45% |
|  | Decrease agricultural exports | Reduce agricultural exports by a specified percentage by the final period. This lever decreases external demand and thus reduces production-related emissions. | 50% |
|  | Expand conservation agriculture | Increase the fraction of cropland managed using conservation agriculture—minimum tillage, permanent soil cover, and crop diversity—while increasing the retention of crop residues. The intervention modifies management factors across crop categories. | Cereals: 8% Fibers: 8% Pulses: 5% Tubers: 5% Vegetables and vines: 5% Other: 8% Magnitude burned: 0% Magnitude removed: 8% |
| Livestock | Decrease exports of livestock | Reduce exports of livestock and livestock products by a specified fraction by the final period, decreasing external demand and associated production. | 50% |
|  | Increase biogas capture at anaerobic manure facilities | Increase the fraction of biogas captured from manure treated in anaerobic facilities, reducing methane emissions. | 90% |
|  | Improve manure management for cattle and pigs | Treat 95% of manure from cattle (dairy and non-dairy) and pigs across various pathways: 37.5% to anaerobic digestion, 12.5% to composting, 25% to daily spread, 12.5% to deep bedding, and 12.5% to solid storage. This reduces unmanaged methane emissions. | Anaerobic digester: 59.4% Composting: 11.9% Daily spread: 23.8 |
| Electricity | Reduce transmission losses | Decrease electrical transmission losses through investments in grid upgrades. The lever sets a maximum transmission loss by 2050; if existing losses are lower they remain unchanged. Transmission losses influence electricity generation requirements. | Transmission loss capped at 6%; minimum possible loss 2%. |
| Industrial Energy | Maximize industrial production efficiency | Reduce energy use through improved industrial production processes (e.g., optimizing throughput, heat integration, recycling waste heat). The transformation increases production efficiency (kJ per tonne). | 40% |
| Transportation | Reduce transport demand | Reduce demand for both public and private transportation through urban planning, non-motorised transport corridors, congestion pricing and other demand management measures. | 75% |
|  | Electrify rail | Increase the share of rail freight and passenger services powered by electricity instead of diesel, shifting baseline energy sources. | 25% |
| Industrial Processes | Demand management (IPPU) | Reduce industrial production levels by a specified fraction, potentially sector-specific. This lever manages demand to reduce resource consumption and emissions. | 30% |
|  | Reduce nitrous oxide emissions | Reduce emission factors of nitrous oxide (N₂O) from industrial processes through improved technologies and abatement measures. | 10% |

Table 4.2: Actions included in Scenario 2

| Subsector | Transformation name | Description | Magnitude Scenario 2 |
| --- | --- | --- | --- |
| Agriculture | Improve rice management | Improve rice cultivation practices to reduce methane emissions by 45%, including better water and fertilizer management, appropriate variety selection, residue management, and optimal seeding. This transformation directly reduces the rice-specific CH4 emission factor. | 45% |
|  | Decrease agricultural exports | Reduce agricultural exports by a specified percentage by the final period. This lever decreases external demand and thus reduces production-related emissions. | 50% |
|  | Reduce supply chain losses | Reduce pre-consumer food losses within the crop supply chain, lowering baseline losses and therefore the quantity of crops produced to meet demand. | 30% |
|  | Expand conservation agriculture | Increase the fraction of cropland managed using conservation agriculture—minimum tillage, permanent soil cover, and crop diversity—while increasing the retention of crop residues. The intervention modifies management factors across crop categories. | Cereals: 8% Fibers: 8% Pulses: 5% Tubers: 5% Vegetables and vines: 5% Other: 8% Magnitude burned: 0% Magnitude removed: 8% |
|  | Improve crop productivity | Apply a fractional increase to crop yields per hectare, enabling the same output on less land and reducing pressure for expansion. | 20% |
| Land Use | Stop deforestation | Halt deforestation of primary and secondary forests by setting transition probabilities from forest to other land uses near zero, conserving carbon stocks. | 100% |
|  | Expand sustainable grazing practices | Reduce loss of soil organic carbon in grasslands by adopting sustainable grazing techniques, thereby increasing the fraction of pastures managed with high carbon sequestration practices. | 95% |
|  | Increase reforestation | Increase the probability that land-use classes transition to secondary forest, effectively increasing forest area through reforestation efforts. | 20% |
|  | Expand silvopasture | Convert portions of pasture to silvopastoral systems by integrating trees with grazing, shifting 30% of pasture area to secondary forest over time while adjusting livestock carrying capacity. | 10% |
| Livestock | Reduce enteric fermentation | Lower methane emissions from enteric fermentation of ruminant livestock by altering feed composition, feed additives or using anti-methanogenic vaccines. Reductions are 40% for buffalo and cattle (dairy and non-dairy) and 56% for goats and sheep. | 50% buffalo 56% cattle 56% cattle nondairy 56% goats 56% sheeps |
|  | Decrease exports of livestock | Reduce exports of livestock and livestock products by a specified fraction by the final period, decreasing external demand and associated production. | 100% |
|  | Increase livestock productivity | Increase livestock carrying capacity (productivity per area) by adopting improved breeds, better feed, and fodder agroforestry on 30–40% of farms. This improves efficiency and reduces emissions per unit of output. | 30% |
|  | Increase biogas capture at anaerobic manure facilities | Increase the fraction of biogas captured from manure treated in anaerobic facilities, reducing methane emissions. | 100% |
|  | Improve manure management for cattle and pigs | Treat 95% of manure from cattle (dairy and non-dairy) and pigs across various pathways: 37.5% to anaerobic digestion, 12.5% to composting, 25% to daily spread, 12.5% to deep bedding, and 12.5% to solid storage. This reduces unmanaged methane emissions. | Anaerobic_digester: 59.4% Composting: 11.9% Daily spread: 23.8 |
|  | Improve manure management for other animals | Treat 95% of manure from sheep, goats, horses, and buffalo using composting, dry lot, daily spread, and solid storage pathways with shares: 37.5% composting, 12.5% dry lot, 25% daily spread, 25% solid storage. This reduces methane and nitrous oxide emissions. | Anaerobic_digester: 59.4% Composting: 11.9% Daily spread: 23.8 |
|  | Improve manure management for poultry | Treat 95% of manure from poultry (chickens) and pigs in specialized poultry manure management systems; all treated manure is directed to poultry-specific facilities. | Anaerobic digester: 47.5% Poultry management:47.5% |
| Soil | Reduce lime application | Decrease the amount of agricultural lime applied to soils by 20%, reducing soil emissions of CO₂. | 40% |
|  | Improve fertilizer application | Reduce total nitrogen fertilizer application through more precise application (e.g., precision agriculture) by 20%, without reducing yields. | 40% |
| Cross | Change diets (healthier diets) | Reduce per capita demand for cattle meat by a specified fraction, also allowing adjustments to certain crop demands (e.g., sugarcane), encouraging healthier diets with lower emissions. | 50% |
|  | Industrial carbon capture and sequestration | Increase adoption of carbon capture and sequestration technologies in non-energy industries. Defaults to 80% of facilities capturing 90% of their process emissions, but parameters can be modified. | 90% |
| Electricity | Reduce transmission losses | Decrease electrical transmission losses through investments in grid upgrades. The lever sets a maximum transmission loss by 2050; if existing losses are lower they remain unchanged. Transmission losses influence electricity generation requirements. | Transmission loss capped at 6%; minimum possible loss 2%. |
|  | Clean hydrogen | Increase the share of hydrogen production derived from green hydrogen (electrolysis). This transformation moves production toward low-carbon hydrogen. | 95% |
|  | 95% renewable electricity | Set a target for electricity generation from renewable sources (solar, wind, hydropower, geothermal, tidal) such that 95% of output is renewable by 2050. Minimum shares for solar, wind and geothermal may apply. | Gas CCS: 49% Geothermal: 5% Nuclear: 5% Solar: 2% Wind: 2% |
| Fugitive Emissions | Minimize leaks | Reduce methane leakage rates across coal, natural gas and oil production, distribution and transmission. The lever specifies a fractional reduction in the CH₄ emission factor for fugitive emissions. | 80% |
|  | Maximize flaring | Increase flaring of gases at orphan wells and mining facilities to convert methane to CO₂, a less potent greenhouse gas. This lever targets vented gas for flaring. | 80% |
| Industrial Energy | Maximize industrial energy efficiency | Improve the average energy efficiency of industrial production processes across fuels, reducing energy demand per unit of output. | 80% |
|  | Maximize industrial production efficiency | Reduce energy use through improved industrial production processes (e.g., optimizing throughput, heat integration, recycling waste heat). The transformation increases production efficiency (kJ per tonne). | 40% |
|  | Fuel switch for high- and low-temp heat | Electrify 90% of low-temperature heat processes and partially electrify high-temperature processes, substituting remaining high-temperature heat with hydrogen. Industry-specific allocation: cement 88% electrical/hydrogen, chemicals 50%, glass 88%, lime and carbonates 88%, metals 92%, paper 18%. This reduces fossil fuel use for industrial heat. | Cement 88%, Chemicals 50%, Glass 88%, Lime & carbonates 88%, Metals 92%, Paper 18%. |
| Stationary Combustion And Other Energy | Reduce building heat demand | Reduce building heat demand via improved thermal envelopes (insulation, windows, sealing) and smart thermostats. This lever decreases heating energy demand. | 50% |
|  | Increase appliance efficiency | Increase the efficiency of electric appliances in buildings (e.g., HVAC, lighting), reducing electricity demand per service delivered. | 50% |
|  | Electrify building heat | Electrify heating and cooking in buildings by replacing fossil-fuel systems with heat pumps and electric stoves. This lever specifies the fraction of heat energy switched to electricity. | 95% |
| Transportation | Reduce transport demand | Reduce demand for both public and private transportation through urban planning, non-motorised transport corridors, congestion pricing and other demand management measures. | 75% |
|  | Increase electric vehicle efficiency | Improve the efficiency of electric vehicles through regulation and technological advancements, resulting in lower energy consumption per kilometre. Efficiency improvement is relative to the final period. | 25% |
|  | Increase non-electric vehicle efficiency | Improve the efficiency of non-electric vehicles (internal combustion engines and other fuels) through better fuel standards and technology, reducing fuel consumption per distance. | 25% |
|  | Increase occupancy for private vehicles | Increase average occupancy rates of private vehicles by implementing carpool lanes, ride-sharing incentives, or other programmes to reduce per capita emissions. | 25% |
|  | Electrify light duty road transport | Increase the share of light duty vehicles (cars, small trucks) powered by electricity. Baseline fossil-fuel shares are reduced proportionally as electrification increases. | 70% |
|  | Fuel switch for maritime transport | Shift maritime transportation from fossil fuels to hydrogen and electricity. A specified fraction of demand is met by hydrogen; the remainder is electrified (adjustable by region). | 70% |
|  | Fuel switch for medium duty transport | Shift medium duty vehicles (heavy freight, regional freight, public buses) from fossil fuels to a mix of electricity and hydrogen. Baseline shares are reduced proportionally. | 70% |
|  | Electrify rail | Increase the share of rail freight and passenger services powered by electricity instead of diesel, shifting baseline energy sources. | 25% |
|  | Mode shift for freight | Shift freight transport modes by moving a share of aviation and road freight demand to rail. Default is 20% of aviation and road freight demand shifted to rail. | 20% |
|  | Mode shift for passenger transport | Shift passenger transport demand from light duty vehicles to other modes (walking, cycling, e-bikes, public transit). Default: 30% of road passenger demand shifts, with breakdown: 17% to walking/human-powered, 33% to powered bikes, 50% to public transport. | 30% |
|  | Mode shift for regional passenger travel | Shift regional passenger travel demand from aviation (10%) and light duty road (20%) to heavier duty road or other modes as specified. This reduces emissions in regional transport. | Aviation: 10% Ligth duty road: 20% |
| Industrial Processes | Reduce cement clinker | Decrease the share of clinker in cement manufacturing by incorporating supplementary cementitious materials (e.g., fly ash, calcined clay) and alternative processes (e.g., LC3). This reduces process and combustion emissions. | 50% |
|  | Demand management (IPPU) | Reduce industrial production levels by a specified fraction, potentially sector-specific. This lever manages demand to reduce resource consumption and emissions. | 30% |
|  | Reduce use of HFCs | Reduce emission factors of hydrofluorocarbons (HFCs) arising from industrial processes by improving refrigerant management, reducing leakages, and switching to alternatives. | 10% |
|  | Reduce nitrous oxide emissions | Reduce emission factors of nitrous oxide (N₂O) from industrial processes through improved technologies and abatement measures. | 10% |
|  | Reduce other fluorinated compounds | Reduce emission factors of other fluorinated compounds (SF₆, NF₃, HCFCs) from industrial processes via improved equipment, leak detection, and replacement. | 10% |
|  | Reduce use of PFCs | Reduce emission factors of perfluorocarbons (PFCs) generated by industrial processes through process changes and substitute materials. | 10% |
| Solid Waste | Consumer food waste reduction | Reduce per capita food waste at the consumer level by a specified fraction, reducing organic waste and associated emissions. | 30% |
|  | Increase composting and biogas | Increase the treatment of organic waste (yard, food, sludge) through anaerobic digestion and composting. Default targets are 47.5% of organic waste to anaerobic digesters and 47.5% to composting. | 47.5% organic waste to biogas 47.5% to compost |
|  | Increase biogas capture (waste) | Increase the fraction of biogas captured from landfills and anaerobic digesters, including installations in institutions like schools. | 85% |
|  | Biogas for energy production | Increase the share of captured biogas that is used for energy generation, displacing fossil fuels. | 85% |
|  | Incineration for energy production | Increase the fraction of solid waste that is incinerated and used for energy, reducing landfill emissions and generating energy. | 85% |
|  | Increase landfilling | Increase the fraction of waste that is not recycled, composted or digested that is sent to sanitary landfills, improving waste management. | 80% |
|  | Increase recycling | Increase the fraction of recyclable waste that is recycled, reducing anaerobic decomposition in landfills and decreasing demand for virgin materials. | 95% |
| Wastewater Treatment | Improved industrial wastewater treatment | Ensure that at least 80% of industrial wastewater is treated in advanced anaerobic facilities, with 10% treated in secondary anaerobic and 10% in secondary aerobic facilities. This reduces emissions and pollution from industrial discharges. | 80% advanced anaerobic 10% secondary anaerobic 10% secondary aerobic. |
|  | Improved rural wastewater treatment | Shift 100% of rural wastewater to treatment via septic tanks, replacing open discharge and other unmanaged pathways. | 10% |
|  | Improved urban wastewater treatment | Shift urban wastewater treatment to a mix of advanced aerobic (30%), advanced anaerobic (30%), secondary anaerobic (20%) and secondary aerobic (20%) facilities, reducing urban pollution. | 30% advanced aerobic 30% advanced anaerobic 20% secondary anaerobic 20% secondary aerobic |
|  | Increase biogas capture at wastewater facilities | Increase the capture of biogas at secondary and tertiary anaerobic wastewater treatment facilities to reduce methane emissions. | 85% |
|  | Increase septic compliance | Improve compliance with septic tank maintenance and pumping schedules in rural areas to ensure full mitigation benefits. | 90% |

Table 4.3: Actions included in Scenario 3

| Subsector | Transformation name | Description | Magnitude |
| --- | --- | --- | --- |
| Agriculture | Improve rice management | Improve rice cultivation practices to reduce methane emissions by 45%, including better water and fertilizer management, appropriate variety selection, residue management, and optimal seeding. This transformation directly reduces the rice-specific CH4 emission factor. | 45% |
|  | Decrease agricultural exports | Reduce agricultural exports by a specified percentage by the final period. This lever decreases external demand and thus reduces production-related emissions. | 50% |
|  | Reduce supply chain losses | Reduce pre-consumer food losses within the crop supply chain, lowering baseline losses and therefore the quantity of crops produced to meet demand. | 30% |
|  | Expand conservation agriculture | Increase the fraction of cropland managed using conservation agriculture—minimum tillage, permanent soil cover, and crop diversity—while increasing the retention of crop residues. The intervention modifies management factors across crop categories. | Cereals: 8% Fibers: 8% Pulses: 5% Tubers: 5% Vegetables and vines: 5% Other: 8% Magnitude burned: 0% Magnitude removed: 8% |
|  | Improve crop productivity | Apply a fractional increase to crop yields per hectare, enabling the same output on less land and reducing pressure for expansion. | 20% |
| Land Use | Stop deforestation | Halt deforestation of primary and secondary forests by setting transition probabilities from forest to other land uses near zero, conserving carbon stocks. | 100% |
|  | Expand sustainable grazing practices | Reduce loss of soil organic carbon in grasslands by adopting sustainable grazing techniques, thereby increasing the fraction of pastures managed with high carbon sequestration practices. | 95% |
|  | Increase reforestation | Increase the probability that land-use classes transition to secondary forest, effectively increasing forest area through reforestation efforts. | 20% |
|  | Expand silvopasture | Convert portions of pasture to silvopastoral systems by integrating trees with grazing, shifting 30% of pasture area to secondary forest over time while adjusting livestock carrying capacity. | 10% |
| Livestock | Reduce enteric fermentation | Lower methane emissions from enteric fermentation of ruminant livestock by altering feed composition, feed additives or using anti-methanogenic vaccines. Reductions are 40% for buffalo and cattle (dairy and non-dairy) and 56% for goats and sheep. | 50% buffalo 56% cattle 56% cattle nondairy 56% goats 56% sheeps |
|  | Decrease exports of livestock | Reduce exports of livestock and livestock products by a specified fraction by the final period, decreasing external demand and associated production. | 100% |
|  | Increase livestock productivity | Increase livestock carrying capacity (productivity per area) by adopting improved breeds, better feed, and fodder agroforestry on 30–40% of farms. This improves efficiency and reduces emissions per unit of output. | 30% |
|  | Increase biogas capture at anaerobic manure facilities | Increase the fraction of biogas captured from manure treated in anaerobic facilities, reducing methane emissions. | 100% |
|  | Improve manure management for cattle and pigs | Treat 95% of manure from cattle (dairy and non-dairy) and pigs across various pathways: 37.5% to anaerobic digestion, 12.5% to composting, 25% to daily spread, 12.5% to deep bedding, and 12.5% to solid storage. This reduces unmanaged methane emissions. | Anaerobic_digester: 59.4% Composting: 11.9% Daily spread: 23.8 |
|  | Improve manure management for other animals | Treat 95% of manure from sheep, goats, horses, and buffalo using composting, dry lot, daily spread, and solid storage pathways with shares: 37.5% composting, 12.5% dry lot, 25% daily spread, 25% solid storage. This reduces methane and nitrous oxide emissions. | Anaerobic_digester: 59.4% Composting: 11.9% Daily spread: 23.8 |
|  | Improve manure management for poultry | Treat 95% of manure from poultry (chickens) and pigs in specialized poultry manure management systems; all treated manure is directed to poultry-specific facilities. | Anaerobic digester: 47.5% Poultry management:47.5% |
| Soil | Reduce lime application | Decrease the amount of agricultural lime applied to soils by 20%, reducing soil emissions of CO₂. | 40% |
|  | Improve fertilizer application | Reduce total nitrogen fertilizer application through more precise application (e.g., precision agriculture) by 20%, without reducing yields. | 40% |
| Cross | Change diets (healthier diets) | Reduce per capita demand for cattle meat by a specified fraction, also allowing adjustments to certain crop demands (e.g., sugarcane), encouraging healthier diets with lower emissions. | 50% |
|  | Industrial carbon capture and sequestration | Increase adoption of carbon capture and sequestration technologies in non-energy industries. Defaults to 80% of facilities capturing 90% of their process emissions, but parameters can be modified. | 90% |
| Ccsq | Increase direct air capture | Scale up Direct Air Capture to remove 50 Mt CO₂e from the atmosphere by 2050, as specified exogenously in installed capacity. This lever adds negative emissions to the system. | 65MT |
| Electricity | Reduce transmission losses | Decrease electrical transmission losses through investments in grid upgrades. The lever sets a maximum transmission loss by 2050; if existing losses are lower they remain unchanged. Transmission losses influence electricity generation requirements. | Transmission loss capped at 6%; minimum possible loss 2%. |
|  | Clean hydrogen | Increase the share of hydrogen production derived from green hydrogen (electrolysis). This transformation moves production toward low-carbon hydrogen. | 95% |
|  | 95% renewable electricity | Set a target for electricity generation from renewable sources (solar, wind, hydropower, geothermal, tidal) such that 95% of output is renewable by 2050. Minimum shares for solar, wind and geothermal may apply. | Gas CCS: 49% Geothermal: 5% Nuclear: 5% Solar: 2% Wind: 2% |
| Fugitive Emissions | Minimize leaks | Reduce methane leakage rates across coal, natural gas and oil production, distribution and transmission. The lever specifies a fractional reduction in the CH₄ emission factor for fugitive emissions. | 80% |
|  | Maximize flaring | Increase flaring of gases at orphan wells and mining facilities to convert methane to CO₂, a less potent greenhouse gas. This lever targets vented gas for flaring. | 80% |
| Industrial Energy | Maximize industrial energy efficiency | Improve the average energy efficiency of industrial production processes across fuels, reducing energy demand per unit of output. | 80% |
|  | Maximize industrial production efficiency | Reduce energy use through improved industrial production processes (e.g., optimizing throughput, heat integration, recycling waste heat). The transformation increases production efficiency (kJ per tonne). | 40% |
|  | Fuel switch for high- and low-temp heat | Electrify 90% of low-temperature heat processes and partially electrify high-temperature processes, substituting remaining high-temperature heat with hydrogen. Industry-specific allocation: cement 88% electrical/hydrogen, chemicals 50%, glass 88%, lime and carbonates 88%, metals 92%, paper 18%. This reduces fossil fuel use for industrial heat. | Cement 88%, Chemicals 50%, Glass 88%, Lime & carbonates 88%, Metals 92%, Paper 18%. |
| Stationary Combustion And Other Energy | Reduce building heat demand | Reduce building heat demand via improved thermal envelopes (insulation, windows, sealing) and smart thermostats. This lever decreases heating energy demand. | 50% |
|  | Increase appliance efficiency | Increase the efficiency of electric appliances in buildings (e.g., HVAC, lighting), reducing electricity demand per service delivered. | 50% |
|  | Electrify building heat | Electrify heating and cooking in buildings by replacing fossil-fuel systems with heat pumps and electric stoves. This lever specifies the fraction of heat energy switched to electricity. | 95% |
| Transportation | Reduce transport demand | Reduce demand for both public and private transportation through urban planning, non-motorised transport corridors, congestion pricing and other demand management measures. | 75% |
|  | Increase electric vehicle efficiency | Improve the efficiency of electric vehicles through regulation and technological advancements, resulting in lower energy consumption per kilometre. Efficiency improvement is relative to the final period. | 25% |
|  | Increase non-electric vehicle efficiency | Improve the efficiency of non-electric vehicles (internal combustion engines and other fuels) through better fuel standards and technology, reducing fuel consumption per distance. | 25% |
|  | Increase occupancy for private vehicles | Increase average occupancy rates of private vehicles by implementing carpool lanes, ride-sharing incentives, or other programmes to reduce per capita emissions. | 25% |
|  | Electrify light duty road transport | Increase the share of light duty vehicles (cars, small trucks) powered by electricity. Baseline fossil-fuel shares are reduced proportionally as electrification increases. | 70% |
|  | Fuel switch for maritime transport | Shift maritime transportation from fossil fuels to hydrogen and electricity. A specified fraction of demand is met by hydrogen; the remainder is electrified (adjustable by region). | 70% |
|  | Fuel switch for medium duty transport | Shift medium duty vehicles (heavy freight, regional freight, public buses) from fossil fuels to a mix of electricity and hydrogen. Baseline shares are reduced proportionally. | 70% |
|  | Electrify rail | Increase the share of rail freight and passenger services powered by electricity instead of diesel, shifting baseline energy sources. | 25% |
|  | Mode shift for freight | Shift freight transport modes by moving a share of aviation and road freight demand to rail. Default is 20% of aviation and road freight demand shifted to rail. | 20% |
|  | Mode shift for passenger transport | Shift passenger transport demand from light duty vehicles to other modes (walking, cycling, e-bikes, public transit). Default: 30% of road passenger demand shifts, with breakdown: 17% to walking/human-powered, 33% to powered bikes, 50% to public transport. | 30% |
|  | Mode shift for regional passenger travel | Shift regional passenger travel demand from aviation (10%) and light duty road (20%) to heavier duty road or other modes as specified. This reduces emissions in regional transport. | Aviation: 10% Ligth duty road: 20% |
| Industrial Processes | Reduce cement clinker | Decrease the share of clinker in cement manufacturing by incorporating supplementary cementitious materials (e.g., fly ash, calcined clay) and alternative processes (e.g., LC3). This reduces process and combustion emissions. | 50% |
|  | Demand management (IPPU) | Reduce industrial production levels by a specified fraction, potentially sector-specific. This lever manages demand to reduce resource consumption and emissions. | 30% |
|  | Reduce use of HFCs | Reduce emission factors of hydrofluorocarbons (HFCs) arising from industrial processes by improving refrigerant management, reducing leakages, and switching to alternatives. | 10% |
|  | Reduce nitrous oxide emissions | Reduce emission factors of nitrous oxide (N₂O) from industrial processes through improved technologies and abatement measures. | 10% |
|  | Reduce other fluorinated compounds | Reduce emission factors of other fluorinated compounds (SF₆, NF₃, HCFCs) from industrial processes via improved equipment, leak detection, and replacement. | 10% |
|  | Reduce use of PFCs | Reduce emission factors of perfluorocarbons (PFCs) generated by industrial processes through process changes and substitute materials. | 10% |
| Solid Waste | Consumer food waste reduction | Reduce per capita food waste at the consumer level by a specified fraction, reducing organic waste and associated emissions. | 30% |
|  | Increase composting and biogas | Increase the treatment of organic waste (yard, food, sludge) through anaerobic digestion and composting. Default targets are 47.5% of organic waste to anaerobic digesters and 47.5% to composting. | 47.5% organic waste to biogas 47.5% to compost |
|  | Increase biogas capture (waste) | Increase the fraction of biogas captured from landfills and anaerobic digesters, including installations in institutions like schools. | 85% |
|  | Biogas for energy production | Increase the share of captured biogas that is used for energy generation, displacing fossil fuels. | 85% |
|  | Incineration for energy production | Increase the fraction of solid waste that is incinerated and used for energy, reducing landfill emissions and generating energy. | 85% |
|  | Increase landfilling | Increase the fraction of waste that is not recycled, composted or digested that is sent to sanitary landfills, improving waste management. | 80% |
|  | Increase recycling | Increase the fraction of recyclable waste that is recycled, reducing anaerobic decomposition in landfills and decreasing demand for virgin materials. | 95% |
| Wastewater Treatment | Improved industrial wastewater treatment | Ensure that at least 80% of industrial wastewater is treated in advanced anaerobic facilities, with 10% treated in secondary anaerobic and 10% in secondary aerobic facilities. This reduces emissions and pollution from industrial discharges. | 80% advanced anaerobic 10% secondary anaerobic 10% secondary aerobic. |
|  | Improved rural wastewater treatment | Shift 100% of rural wastewater to treatment via septic tanks, replacing open discharge and other unmanaged pathways. | 10% |
|  | Improved urban wastewater treatment | Shift urban wastewater treatment to a mix of advanced aerobic (30%), advanced anaerobic (30%), secondary anaerobic (20%) and secondary aerobic (20%) facilities, reducing urban pollution. | 30% advanced aerobic 30% advanced anaerobic 20% secondary anaerobic 20% secondary aerobic |
|  | Increase biogas capture at wastewater facilities | Increase the capture of biogas at secondary and tertiary anaerobic wastewater treatment facilities to reduce methane emissions. | 85% |
|  | Increase septic compliance | Improve compliance with septic tank maintenance and pumping schedules in rural areas to ensure full mitigation benefits. | 90% |

Table 4.4: Actions included in Scenario 4

| Subsector | Transformation name | Description | Magnitude |
| --- | --- | --- | --- |
| Agriculture | Improve rice management | Improve rice cultivation practices to reduce methane emissions by 45%, including better water and fertilizer management, appropriate variety selection, residue management, and optimal seeding. This transformation directly reduces the rice-specific CH4 emission factor. | 45% |
|  | Decrease agricultural exports | Reduce agricultural exports by a specified percentage by the final period. This lever decreases external demand and thus reduces production-related emissions. | 50% |
|  | Reduce supply chain losses | Reduce pre-consumer food losses within the crop supply chain, lowering baseline losses and therefore the quantity of crops produced to meet demand. | 30% |
|  | Expand conservation agriculture | Increase the fraction of cropland managed using conservation agriculture—minimum tillage, permanent soil cover, and crop diversity—while increasing the retention of crop residues. The intervention modifies management factors across crop categories. | Cereals: 8% Fibers: 8% Pulses: 5% Tubers: 5% Vegetables and vines: 5% Other: 8% Magnitude burned: 0% Magnitude removed: 8% |
|  | Improve crop productivity | Apply a fractional increase to crop yields per hectare, enabling the same output on less land and reducing pressure for expansion. | 20% |
| Land Use | Stop deforestation | Halt deforestation of primary and secondary forests by setting transition probabilities from forest to other land uses near zero, conserving carbon stocks. | 100% |
|  | Expand sustainable grazing practices | Reduce loss of soil organic carbon in grasslands by adopting sustainable grazing techniques, thereby increasing the fraction of pastures managed with high carbon sequestration practices. | 95% |
|  | Increase reforestation | Increase the probability that land-use classes transition to secondary forest, effectively increasing forest area through reforestation efforts. | 20% |
|  | Expand silvopasture | Convert portions of pasture to silvopastoral systems by integrating trees with grazing, shifting 30% of pasture area to secondary forest over time while adjusting livestock carrying capacity. | 10% |
| Livestock | Reduce enteric fermentation | Lower methane emissions from enteric fermentation of ruminant livestock by altering feed composition, feed additives or using anti-methanogenic vaccines. Reductions are 40% for buffalo and cattle (dairy and non-dairy) and 56% for goats and sheep. | 50% buffalo 56% cattle 56% cattle nondairy 56% goats 56% sheeps |
|  | Decrease exports of livestock | Reduce exports of livestock and livestock products by a specified fraction by the final period, decreasing external demand and associated production. | 100% |
|  | Increase livestock productivity | Increase livestock carrying capacity (productivity per area) by adopting improved breeds, better feed, and fodder agroforestry on 30–40% of farms. This improves efficiency and reduces emissions per unit of output. | 30% |
|  | Increase biogas capture at anaerobic manure facilities | Increase the fraction of biogas captured from manure treated in anaerobic facilities, reducing methane emissions. | 100% |
|  | Improve manure management for cattle and pigs | Treat 95% of manure from cattle (dairy and non-dairy) and pigs across various pathways: 37.5% to anaerobic digestion, 12.5% to composting, 25% to daily spread, 12.5% to deep bedding, and 12.5% to solid storage. This reduces unmanaged methane emissions. | Anaerobic_digester: 59.4% Composting: 11.9% Daily spread: 23.8 |
|  | Improve manure management for other animals | Treat 95% of manure from sheep, goats, horses, and buffalo using composting, dry lot, daily spread, and solid storage pathways with shares: 37.5% composting, 12.5% dry lot, 25% daily spread, 25% solid storage. This reduces methane and nitrous oxide emissions. | Anaerobic_digester: 59.4% Composting: 11.9% Daily spread: 23.8 |
|  | Improve manure management for poultry | Treat 95% of manure from poultry (chickens) and pigs in specialized poultry manure management systems; all treated manure is directed to poultry-specific facilities. | Anaerobic digester: 47.5% Poultry management:47.5% |
| Soil | Reduce lime application | Decrease the amount of agricultural lime applied to soils by 20%, reducing soil emissions of CO₂. | 40% |
|  | Improve fertilizer application | Reduce total nitrogen fertilizer application through more precise application (e.g., precision agriculture) by 20%, without reducing yields. | 40% |
| Cross | Change diets (healthier diets) | Reduce per capita demand for cattle meat by a specified fraction, also allowing adjustments to certain crop demands (e.g., sugarcane), encouraging healthier diets with lower emissions. | 50% |
|  | Industrial carbon capture and sequestration | Increase adoption of carbon capture and sequestration technologies in non-energy industries. Defaults to 80% of facilities capturing 90% of their process emissions, but parameters can be modified. | 90% |
| Ccsq | Increase direct air capture | Scale up Direct Air Capture to remove 50 Mt CO₂e from the atmosphere by 2050, as specified exogenously in installed capacity. This lever adds negative emissions to the system. | 81MT |
| Electricity | Reduce transmission losses | Decrease electrical transmission losses through investments in grid upgrades. The lever sets a maximum transmission loss by 2050; if existing losses are lower they remain unchanged. Transmission losses influence electricity generation requirements. | Transmission loss capped at 6%; minimum possible loss 2%. |
|  | Clean hydrogen | Increase the share of hydrogen production derived from green hydrogen (electrolysis). This transformation moves production toward low-carbon hydrogen. | 95% |
|  | 95% renewable electricity | Set a target for electricity generation from renewable sources (solar, wind, hydropower, geothermal, tidal) such that 95% of output is renewable by 2050. Minimum shares for solar, wind and geothermal may apply. | Gas CCS: 49% Geothermal: 5% Nuclear: 5% Solar: 2% Wind: 2% |
| Fugitive Emissions | Minimize leaks | Reduce methane leakage rates across coal, natural gas and oil production, distribution and transmission. The lever specifies a fractional reduction in the CH₄ emission factor for fugitive emissions. | 80% |
|  | Maximize flaring | Increase flaring of gases at orphan wells and mining facilities to convert methane to CO₂, a less potent greenhouse gas. This lever targets vented gas for flaring. | 80% |
| Industrial Energy | Maximize industrial energy efficiency | Improve the average energy efficiency of industrial production processes across fuels, reducing energy demand per unit of output. | 80% |
|  | Maximize industrial production efficiency | Reduce energy use through improved industrial production processes (e.g., optimizing throughput, heat integration, recycling waste heat). The transformation increases production efficiency (kJ per tonne). | 40% |
|  | Fuel switch for high- and low-temp heat | Electrify 90% of low-temperature heat processes and partially electrify high-temperature processes, substituting remaining high-temperature heat with hydrogen. Industry-specific allocation: cement 88% electrical/hydrogen, chemicals 50%, glass 88%, lime and carbonates 88%, metals 92%, paper 18%. This reduces fossil fuel use for industrial heat. | Cement 88%, Chemicals 50%, Glass 88%, Lime & carbonates 88%, Metals 92%, Paper 18%. |
| Stationary Combustion And Other Energy | Reduce building heat demand | Reduce building heat demand via improved thermal envelopes (insulation, windows, sealing) and smart thermostats. This lever decreases heating energy demand. | 50% |
|  | Increase appliance efficiency | Increase the efficiency of electric appliances in buildings (e.g., HVAC, lighting), reducing electricity demand per service delivered. | 50% |
|  | Electrify building heat | Electrify heating and cooking in buildings by replacing fossil-fuel systems with heat pumps and electric stoves. This lever specifies the fraction of heat energy switched to electricity. | 95% |
| Transportation | Reduce transport demand | Reduce demand for both public and private transportation through urban planning, non-motorised transport corridors, congestion pricing and other demand management measures. | 75% |
|  | Increase electric vehicle efficiency | Improve the efficiency of electric vehicles through regulation and technological advancements, resulting in lower energy consumption per kilometre. Efficiency improvement is relative to the final period. | 25% |
|  | Increase non-electric vehicle efficiency | Improve the efficiency of non-electric vehicles (internal combustion engines and other fuels) through better fuel standards and technology, reducing fuel consumption per distance. | 25% |
|  | Increase occupancy for private vehicles | Increase average occupancy rates of private vehicles by implementing carpool lanes, ride-sharing incentives, or other programmes to reduce per capita emissions. | 25% |
|  | Electrify light duty road transport | Increase the share of light duty vehicles (cars, small trucks) powered by electricity. Baseline fossil-fuel shares are reduced proportionally as electrification increases. | 70% |
|  | Fuel switch for maritime transport | Shift maritime transportation from fossil fuels to hydrogen and electricity. A specified fraction of demand is met by hydrogen; the remainder is electrified (adjustable by region). | 70% |
|  | Fuel switch for medium duty transport | Shift medium duty vehicles (heavy freight, regional freight, public buses) from fossil fuels to a mix of electricity and hydrogen. Baseline shares are reduced proportionally. | 70% |
|  | Electrify rail | Increase the share of rail freight and passenger services powered by electricity instead of diesel, shifting baseline energy sources. | 25% |
|  | Mode shift for freight | Shift freight transport modes by moving a share of aviation and road freight demand to rail. Default is 20% of aviation and road freight demand shifted to rail. | 20% |
|  | Mode shift for passenger transport | Shift passenger transport demand from light duty vehicles to other modes (walking, cycling, e-bikes, public transit). Default: 30% of road passenger demand shifts, with breakdown: 17% to walking/human-powered, 33% to powered bikes, 50% to public transport. | 30% |
|  | Mode shift for regional passenger travel | Shift regional passenger travel demand from aviation (10%) and light duty road (20%) to heavier duty road or other modes as specified. This reduces emissions in regional transport. | Aviation: 10% Ligth duty road: 20% |
| Industrial Processes | Reduce cement clinker | Decrease the share of clinker in cement manufacturing by incorporating supplementary cementitious materials (e.g., fly ash, calcined clay) and alternative processes (e.g., LC3). This reduces process and combustion emissions. | 50% |
|  | Demand management (IPPU) | Reduce industrial production levels by a specified fraction, potentially sector-specific. This lever manages demand to reduce resource consumption and emissions. | 30% |
|  | Reduce use of HFCs | Reduce emission factors of hydrofluorocarbons (HFCs) arising from industrial processes by improving refrigerant management, reducing leakages, and switching to alternatives. | 10% |
|  | Reduce nitrous oxide emissions | Reduce emission factors of nitrous oxide (N₂O) from industrial processes through improved technologies and abatement measures. | 10% |
|  | Reduce other fluorinated compounds | Reduce emission factors of other fluorinated compounds (SF₆, NF₃, HCFCs) from industrial processes via improved equipment, leak detection, and replacement. | 10% |
|  | Reduce use of PFCs | Reduce emission factors of perfluorocarbons (PFCs) generated by industrial processes through process changes and substitute materials. | 10% |
| Solid Waste | Consumer food waste reduction | Reduce per capita food waste at the consumer level by a specified fraction, reducing organic waste and associated emissions. | 30% |
|  | Increase composting and biogas | Increase the treatment of organic waste (yard, food, sludge) through anaerobic digestion and composting. Default targets are 47.5% of organic waste to anaerobic digesters and 47.5% to composting. | 47.5% organic waste to biogas 47.5% to compost |
|  | Increase biogas capture (waste) | Increase the fraction of biogas captured from landfills and anaerobic digesters, including installations in institutions like schools. | 85% |
|  | Biogas for energy production | Increase the share of captured biogas that is used for energy generation, displacing fossil fuels. | 85% |
|  | Incineration for energy production | Increase the fraction of solid waste that is incinerated and used for energy, reducing landfill emissions and generating energy. | 85% |
|  | Increase landfilling | Increase the fraction of waste that is not recycled, composted or digested that is sent to sanitary landfills, improving waste management. | 80% |
|  | Increase recycling | Increase the fraction of recyclable waste that is recycled, reducing anaerobic decomposition in landfills and decreasing demand for virgin materials. | 95% |
| Wastewater Treatment | Improved industrial wastewater treatment | Ensure that at least 80% of industrial wastewater is treated in advanced anaerobic facilities, with 10% treated in secondary anaerobic and 10% in secondary aerobic facilities. This reduces emissions and pollution from industrial discharges. | 80% advanced anaerobic 10% secondary anaerobic 10% secondary aerobic. |
|  | Improved rural wastewater treatment | Shift 100% of rural wastewater to treatment via septic tanks, replacing open discharge and other unmanaged pathways. | 10% |
|  | Improved urban wastewater treatment | Shift urban wastewater treatment to a mix of advanced aerobic (30%), advanced anaerobic (30%), secondary anaerobic (20%) and secondary aerobic (20%) facilities, reducing urban pollution. | 30% advanced aerobic 30% advanced anaerobic 20% secondary anaerobic 20% secondary aerobic |
|  | Increase biogas capture at wastewater facilities | Increase the capture of biogas at secondary and tertiary anaerobic wastewater treatment facilities to reduce methane emissions. | 85% |
|  | Increase septic compliance | Improve compliance with septic tank maintenance and pumping schedules in rural areas to ensure full mitigation benefits. | 90% |

Table 4.5: Actions included in Scenario 5

| Subsector | Transformation name | Description | Magnitude |
| --- | --- | --- | --- |
| Agriculture | Improve rice management | Improve rice cultivation practices to reduce methane emissions by 45%, including better water and fertilizer management, appropriate variety selection, residue management, and optimal seeding. This transformation directly reduces the rice-specific CH4 emission factor. | 45% |
|  | Decrease agricultural exports | Reduce agricultural exports by a specified percentage by the final period. This lever decreases external demand and thus reduces production-related emissions. | 50% |
|  | Reduce supply chain losses | Reduce pre-consumer food losses within the crop supply chain, lowering baseline losses and therefore the quantity of crops produced to meet demand. | 30% |
|  | Expand conservation agriculture | Increase the fraction of cropland managed using conservation agriculture—minimum tillage, permanent soil cover, and crop diversity—while increasing the retention of crop residues. The intervention modifies management factors across crop categories. | Cereals: 8% Fibers: 8% Pulses: 5% Tubers: 5% Vegetables and vines: 5% Other: 8% Magnitude burned: 0% Magnitude removed: 8% |
|  | Improve crop productivity | Apply a fractional increase to crop yields per hectare, enabling the same output on less land and reducing pressure for expansion. | 20% |
| Land Use | Stop deforestation | Halt deforestation of primary and secondary forests by setting transition probabilities from forest to other land uses near zero, conserving carbon stocks. | 100% |
|  | Expand sustainable grazing practices | Reduce loss of soil organic carbon in grasslands by adopting sustainable grazing techniques, thereby increasing the fraction of pastures managed with high carbon sequestration practices. | 95% |
|  | Increase reforestation | Increase the probability that land-use classes transition to secondary forest, effectively increasing forest area through reforestation efforts. | 20% |
|  | Expand silvopasture | Convert portions of pasture to silvopastoral systems by integrating trees with grazing, shifting 30% of pasture area to secondary forest over time while adjusting livestock carrying capacity. | 10% |
| Livestock | Reduce enteric fermentation | Lower methane emissions from enteric fermentation of ruminant livestock by altering feed composition, feed additives or using anti-methanogenic vaccines. Reductions are 40% for buffalo and cattle (dairy and non-dairy) and 56% for goats and sheep. | 50% buffalo 56% cattle 56% cattle nondairy 56% goats 56% sheeps |
|  | Decrease exports of livestock | Reduce exports of livestock and livestock products by a specified fraction by the final period, decreasing external demand and associated production. | 100% |
|  | Increase livestock productivity | Increase livestock carrying capacity (productivity per area) by adopting improved breeds, better feed, and fodder agroforestry on 30–40% of farms. This improves efficiency and reduces emissions per unit of output. | 30% |
|  | Increase biogas capture at anaerobic manure facilities | Increase the fraction of biogas captured from manure treated in anaerobic facilities, reducing methane emissions. | 100% |
|  | Improve manure management for cattle and pigs | Treat 95% of manure from cattle (dairy and non-dairy) and pigs across various pathways: 37.5% to anaerobic digestion, 12.5% to composting, 25% to daily spread, 12.5% to deep bedding, and 12.5% to solid storage. This reduces unmanaged methane emissions. | Anaerobic_digester: 59.4% Composting: 11.9% Daily spread: 23.8 |
|  | Improve manure management for other animals | Treat 95% of manure from sheep, goats, horses, and buffalo using composting, dry lot, daily spread, and solid storage pathways with shares: 37.5% composting, 12.5% dry lot, 25% daily spread, 25% solid storage. This reduces methane and nitrous oxide emissions. | Anaerobic_digester: 59.4% Composting: 11.9% Daily spread: 23.8 |
|  | Improve manure management for poultry | Treat 95% of manure from poultry (chickens) and pigs in specialized poultry manure management systems; all treated manure is directed to poultry-specific facilities. | Anaerobic digester: 47.5% Poultry management:47.5% |
| Soil | Reduce lime application | Decrease the amount of agricultural lime applied to soils by 20%, reducing soil emissions of CO₂. | 40% |
|  | Improve fertilizer application | Reduce total nitrogen fertilizer application through more precise application (e.g., precision agriculture) by 20%, without reducing yields. | 40% |
| Cross | Change diets (healthier diets) | Reduce per capita demand for cattle meat by a specified fraction, also allowing adjustments to certain crop demands (e.g., sugarcane), encouraging healthier diets with lower emissions. | 50% |
|  | Industrial carbon capture and sequestration | Increase adoption of carbon capture and sequestration technologies in non-energy industries. Defaults to 80% of facilities capturing 90% of their process emissions, but parameters can be modified. | 90% |
| CCSQ | Increase direct air capture | Scale up Direct Air Capture to remove 50 Mt CO₂e from the atmosphere by 2050, as specified exogenously in installed capacity. This lever adds negative emissions to the system. | 32MT |
| Electricity | Reduce transmission losses | Decrease electrical transmission losses through investments in grid upgrades. The lever sets a maximum transmission loss by 2050; if existing losses are lower they remain unchanged. Transmission losses influence electricity generation requirements. | Transmission loss capped at 6%; minimum possible loss 2%. |
|  | Clean hydrogen | Increase the share of hydrogen production derived from green hydrogen (electrolysis). This transformation moves production toward low-carbon hydrogen. | 95% |
|  | 95% renewable electricity | Set a target for electricity generation from renewable sources (solar, wind, hydropower, geothermal, tidal) such that 95% of output is renewable by 2050. Minimum shares for solar, wind and geothermal may apply. | Geothermal: 15% Nuclear: 5% Solar: 49% Wind: 3% |
| Fugitive emissions | Minimize leaks | Reduce methane leakage rates across coal, natural gas and oil production, distribution and transmission. The lever specifies a fractional reduction in the CH₄ emission factor for fugitive emissions. | 80% |
|  | Maximize flaring | Increase flaring of gases at orphan wells and mining facilities to convert methane to CO₂, a less potent greenhouse gas. This lever targets vented gas for flaring. | 80% |
| Industrial energy | Maximize industrial energy efficiency | Improve the average energy efficiency of industrial production processes across fuels, reducing energy demand per unit of output. | 80% |
|  | Maximize industrial production efficiency | Reduce energy use through improved industrial production processes (e.g., optimizing throughput, heat integration, recycling waste heat). The transformation increases production efficiency (kJ per tonne). | 40% |
|  | Fuel switch for high- and low-temp heat | Electrify 90% of low-temperature heat processes and partially electrify high-temperature processes, substituting remaining high-temperature heat with hydrogen. Industry-specific allocation: cement 88% electrical/hydrogen, chemicals 50%, glass 88%, lime and carbonates 88%, metals 92%, paper 18%. This reduces fossil fuel use for industrial heat. | Cement 88%, Chemicals 50%, Glass 88%, Lime & carbonates 88%, Metals 92%, Paper 18%. |
| Stationary combustion and other energy | Reduce building heat demand | Reduce building heat demand via improved thermal envelopes (insulation, windows, sealing) and smart thermostats. This lever decreases heating energy demand. | 50% |
|  | Increase appliance efficiency | Increase the efficiency of electric appliances in buildings (e.g., HVAC, lighting), reducing electricity demand per service delivered. | 50% |
|  | Electrify building heat | Electrify heating and cooking in buildings by replacing fossil-fuel systems with heat pumps and electric stoves. This lever specifies the fraction of heat energy switched to electricity. | 95% |
| Transportation | Reduce transport demand | Reduce demand for both public and private transportation through urban planning, non-motorised transport corridors, congestion pricing and other demand management measures. | 75% |
|  | Increase electric vehicle efficiency | Improve the efficiency of electric vehicles through regulation and technological advancements, resulting in lower energy consumption per kilometre. Efficiency improvement is relative to the final period. | 25% |
|  | Increase non-electric vehicle efficiency | Improve the efficiency of non-electric vehicles (internal combustion engines and other fuels) through better fuel standards and technology, reducing fuel consumption per distance. | 25% |
|  | Increase occupancy for private vehicles | Increase average occupancy rates of private vehicles by implementing carpool lanes, ride-sharing incentives, or other programmes to reduce per capita emissions. | 25% |
|  | Electrify light duty road transport | Increase the share of light duty vehicles (cars, small trucks) powered by electricity. Baseline fossil-fuel shares are reduced proportionally as electrification increases. | 70% |
|  | Fuel switch for maritime transport | Shift maritime transportation from fossil fuels to hydrogen and electricity. A specified fraction of demand is met by hydrogen; the remainder is electrified (adjustable by region). | 70% |
|  | Fuel switch for medium duty transport | Shift medium duty vehicles (heavy freight, regional freight, public buses) from fossil fuels to a mix of electricity and hydrogen. Baseline shares are reduced proportionally. | 70% |
|  | Electrify rail | Increase the share of rail freight and passenger services powered by electricity instead of diesel, shifting baseline energy sources. | 25% |
|  | Mode shift for freight | Shift freight transport modes by moving a share of aviation and road freight demand to rail. Default is 20% of aviation and road freight demand shifted to rail. | 20% |
|  | Mode shift for passenger transport | Shift passenger transport demand from light duty vehicles to other modes (walking, cycling, e-bikes, public transit). Default: 30% of road passenger demand shifts, with breakdown: 17% to walking/human-powered, 33% to powered bikes, 50% to public transport. | 30% |
|  | Mode shift for regional passenger travel | Shift regional passenger travel demand from aviation (10%) and light duty road (20%) to heavier duty road or other modes as specified. This reduces emissions in regional transport. | Aviation: 10% Ligth duty road: 20% |
| Industrial Processes | Reduce cement clinker | Decrease the share of clinker in cement manufacturing by incorporating supplementary cementitious materials (e.g., fly ash, calcined clay) and alternative processes (e.g., LC3). This reduces process and combustion emissions. | 50% |
|  | Demand management (IPPU) | Reduce industrial production levels by a specified fraction, potentially sector-specific. This lever manages demand to reduce resource consumption and emissions. | 30% |
|  | Reduce use of HFCs | Reduce emission factors of hydrofluorocarbons (HFCs) arising from industrial processes by improving refrigerant management, reducing leakages, and switching to alternatives. | 10% |
|  | Reduce nitrous oxide emissions | Reduce emission factors of nitrous oxide (N₂O) from industrial processes through improved technologies and abatement measures. | 10% |
|  | Reduce other fluorinated compounds | Reduce emission factors of other fluorinated compounds (SF₆, NF₃, HCFCs) from industrial processes via improved equipment, leak detection, and replacement. | 10% |
|  | Reduce use of PFCs | Reduce emission factors of perfluorocarbons (PFCs) generated by industrial processes through process changes and substitute materials. | 10% |
| Solid Waste | Consumer food waste reduction | Reduce per capita food waste at the consumer level by a specified fraction, reducing organic waste and associated emissions. | 30% |
|  | Increase composting and biogas | Increase the treatment of organic waste (yard, food, sludge) through anaerobic digestion and composting. Default targets are 47.5% of organic waste to anaerobic digesters and 47.5% to composting. | 47.5% organic waste to biogas 47.5% to compost |
|  | Increase biogas capture (waste) | Increase the fraction of biogas captured from landfills and anaerobic digesters, including installations in institutions like schools. | 85% |
|  | Biogas for energy production | Increase the share of captured biogas that is used for energy generation, displacing fossil fuels. | 85% |
|  | Incineration for energy production | Increase the fraction of solid waste that is incinerated and used for energy, reducing landfill emissions and generating energy. | 85% |
|  | Increase landfilling | Increase the fraction of waste that is not recycled, composted or digested that is sent to sanitary landfills, improving waste management. | 80% |
|  | Increase recycling | Increase the fraction of recyclable waste that is recycled, reducing anaerobic decomposition in landfills and decreasing demand for virgin materials. | 95% |
| Wastewater Treatment | Improved industrial wastewater treatment | Ensure that at least 80% of industrial wastewater is treated in advanced anaerobic facilities, with 10% treated in secondary anaerobic and 10% in secondary aerobic facilities. This reduces emissions and pollution from industrial discharges. | 80% advanced anaerobic 10% secondary anaerobic 10% secondary aerobic. |
|  | Improved rural wastewater treatment | Shift 100% of rural wastewater to treatment via septic tanks, replacing open discharge and other unmanaged pathways. | 10% |
|  | Improved urban wastewater treatment | Shift urban wastewater treatment to a mix of advanced aerobic (30%), advanced anaerobic (30%), secondary anaerobic (20%) and secondary aerobic (20%) facilities, reducing urban pollution. | 30% advanced aerobic 30% advanced anaerobic 20% secondary anaerobic 20% secondary aerobic |
|  | Increase biogas capture at wastewater facilities | Increase the capture of biogas at secondary and tertiary anaerobic wastewater treatment facilities to reduce methane emissions. | 85% |
|  | Increase septic compliance | Improve compliance with septic tank maintenance and pumping schedules in rural areas to ensure full mitigation benefits. | 90% |